ontario regulation 194/06

made under the

mining act

Made: May 3, 2006
Filed: May 10, 2006
Published on e-Laws: May 11, 2006
Printed in The Ontario Gazette: May 27, 2006

Amending O. Reg. 240/00

(Mine Development and Closure under Part VII of the Act)

1.  Paragraph 5 of subsection 3 (1) of Ontario Regulation 240/00 is revoked and the following substituted:

5.	Surface stripping on mining lands where the surface area over which the surface stripping is carried out is greater than 10,000 square metres, or where the volume of surface stripping is greater than 10,000 cubic metres, except where all of the following are satisfied:

i.	Surface stripping is carried out in two or more separate areas on the mining lands.

ii.	The edges of each area where surface stripping is carried out are separated by a minimum of 500 metres.

iii.	In each area where surface stripping is carried out,

A.	the surface area over which the surface stripping is carried out is not greater than 10,000 square metres, and

B.	the volume of surface stripping is not greater than 10,000 cubic metres.

2.  (1)  Subsection 8 (1) of the Regulation is amended by striking out “and” at the end of clause (a), by adding “and” at the end of clause (b), and by adding the following clause:

(c)	by notifying directly the owners of the lands that are adjacent to the project of the public information session.

(2)  Section 8 of the Regulation is amended by adding the following subsection:

(4)  The proponent shall provide the Director with the names of the owners of lands adjacent to the project notified of the public information session.

3.  Subsection 12 (4) of the Regulation is amended by adding the following clause:

(a.1)	provide a statement of the specific aspects of the closure plan with respect to which the certificate relates;

4.  (1)  The definition of “life of a mine” in subsection 15 (1) of the Regulation is revoked and the following substituted:

“life of a mine” means the projected length of time that a project will be in mine production, be processing mineral products resulting from mine production or the active placement of tailings;

(2)  Subsection 15 (4) of the Regulation is amended by striking out the portion before clause (a) and substituting the following:

(4)  Subject to subsections (4.1) and (4.2), the life of a mine is calculated with reference to,

.     .     .     .     .

(3)  Section 15 of the Regulation is amended by adding the following subsections:

(4.1)  For those projects, located on their own site, that process mineral products resulting from mine production such as but not limited to refining, smelting or milling, the life of a mine is calculated with reference to total feed dedicated to the processing facility over the life of the contributing mines divided by the annual production schedule if,

(a)	at least two thirds of mined material supplied to the processing facility during the proponent’s fiscal year came from the proponent’s proportioned share of mined material from mines in which it has an ownership interest; and

(b)	at least one third of mined material supplied to the processing facility during the proponent’s fiscal year came from the proponent’s proportioned share of mined material from Ontario mines.

(4.2)  For those projects that are located on their own site and consist of the active placement of tailings, the life of the mine is calculated with reference to total feed dedicated to the processing facility producing the tailings over the life of all the proponent’s contributing mines divided by the annual production schedule of the processing facility if,

(a)	at least two thirds of mined material supplied to the processing facility during the proponent’s fiscal year came from the proponent’s proportioned share of mined material from mines in which it has an ownership interest; and

(b)	at least one third of mined material supplied to the processing facility during the proponent’s fiscal year came from the proponent’s proportioned share of mined material from Ontario mines.

5.  Paragraph 5 of subsection 24 (2) of the Regulation is amended by adding “waterlines” after “pipelines”.

6.  (1)  Section 16 of Schedule 1 to the Regulation is amended by adding “ramps, declines or portals” after “adits”.

(2)  Clause 24 (a) of Schedule 1 to the Regulation is amended by striking out “1.25” and substituting “2.0”.

(3)  Section 26 of Schedule 1 to the Regulation is revoked and the following substituted:

26.  If fencing is used, fences,

(a)	shall be constructed of at least #6 gauge chain-link galvanized material;

(b)	shall be secured against access at the bottom;

(c)	where no geotechnical study exists, shall be set back from the toe of the pit at least a distance equivalent to the pit depth so as to locate the fence beyond any area of potential pit instability; and

(d)	shall be used in conjunction with signs.

(4)  Subsection 33 (2) of Schedule 1 to the Regulation is amended by adding “justified” before “documented”.

(5)  Section 33 of Schedule 1 to the Regulation is amended by adding the following subsection:

(3)  If it is determined that the appropriate rehabilitation measures include the use of fencing, the fences shall be constructed in accordance with the requirements set out in section 26 of this Schedule.

(6)  Subsection 34 (3) of Schedule 1 to the Regulation is amended by adding “justified” before “documented”.

(7)  Section 34 of Schedule 1 to the Regulation is amended by adding the following subsection:

(4)  If it is determined that the appropriate rehabilitation measures include the use of  fencing, the fences shall be constructed in accordance with the requirements set out in section 26 of this Schedule.

(8)  Subsection 57 (2) of Schedule 1 to the Regulation is amended by striking out the portion before paragraph 1 and substituting the following:

(2)  The sampling program shall be undertaken in accordance with both of the following documents by a person who is qualified in Ontario as a professional geoscientist or agrologist or as a qualified professional engineer in Ontario:

.     .     .     .     .

(9)  Section 58 of Schedule 1 to the Regulation is revoked and the following substituted:

Testing and Interpretation

58.  Testing of the materials sampled pursuant to section 57 of this Schedule and the interpretation of the data shall be conducted in accordance with the documents listed in subsection 57 (2), and shall be certified by a person who is qualified as a professional geoscientist or agrologist or as a professional engineer having geological and geochemical experience.

(10)  Section 59 of Schedule 1 to the Regulation is amended by adding the following subsections:

(3)  In order to meet the objectives of the closure plan, wet and dry covers for materials that have ML or ARD potential shall be designed and constructed in accordance with best engineering practices and be certified by a qualified professional engineer.

(4)  Analytical models shall be used to predict the performance of wet and dry covers for the materials mentioned in subsection (3) and a monitoring program shall be put in place to test the results predicted by the analytical models used.

7.  (1)  Item 3 of Schedule 2 to the Regulation is amended by adding the following subclause in Column 2:

| Item | Column 1 | Column 2 |
| --- | --- | --- |
|  |  | (iv) plans and sections of proposed new underground development. |

(2)  Item 9 of Schedule 2 to the Regulation is amended by adding “including reports of all studies conducted under sections 30, 31 and 32 of Schedule 1” at the end of subclause (iv) in Column 2.

(3)  Item 14 of Schedule 2 to the Regulation is amended by adding “proposed” before “closure plan” at the end of subclause (i) in Column 2.

8.  This Regulation comes into force 90 days after it is filed.

Back to top